From Portha Paper:

dFjdm=∑νijri

Where Fj is molar flow rate (mol/s), νij is stoichiometric coefficient in reaction I and ri is rate in reaction I, m is the mass of catalyst

Convert this formulation to be per unit reactor length (z) assuming constant area reactor

Fj=Nj=NjVAuz=njAuz   mols

dm=ρcAdz

dFjdm=1ρcdnjdzu+njdudz

I’ve ignored what exactly “area” means right now. Let’s assume it is the pipe area, so then ρc is the “apparent” catalyst volume and u(z) is the average velocity in the pipe assuming no catalyst present

By mass conservation:

ρgAu=const

duu=-dρgρg

Therefore:

dFjdm=1ρcdnjdzu-njuρgdρgdz

Now use nj=nxj and factor out common terms.

dFjdm=nuρcdxjdz-xjρgdρgdz

Replace ρg=nV∑xiMWi=nVMW and simplify:

dFjdm=nuρcdxjdz-xjMW MWkdxkdz

Re-write the RHS as f=∑νijri

Governing equation is now:

Mdxdz=ρcnuf(x)

Where:

Mjk=δjk-xjMWMWk

From mass conservation:

Γ=nu

m=ΓzMWzA=const

ΓzMWz=Γ0MW0

Γz=Γ0MW0MWz=ρ0u0MW(z)

Governing equation becomes:

Mdxdz=ρc MWzρ0u0f(x)

This has a degenerate mass matrix. It can be fixed by replacing one of the equations with ∑xj=1

Now convert to a constant volume reactor.

u=dzdt

(Naïve approach – need to double check if this is consistent, but pretty sure it is PROVIDED I have handled the area’s correctly) sub the following in:

udt=dz

To get:

Mudxdt=ρcnufx,z

Mdxdt=ρcnfx, t

Re-phrase in terms of pressure and temperature:

Mdxdt=ρcRuTPfx, t, T, P

For a batch reactor, it makes more sense to work with total mole number, in which case:

Mdxdt=mcRuTPfx, t, T, P

For constant T, P this is easy. Otherwise, couple to energy equation and EoS.

Now I want to get rid of the degenerate mass matrix. Replace equation N with xk=1. This is now a DAE, which is a different kind of degenerate mass matrix, but one that the solvers know how to handle.

Alternately, just sub in the constraint equation xN=1-xi≠N

xjMW MWkdxkdz=xjMW MWk-MWNdxk≠Ndz

This gives a mass matrix with elements:

Mjk=δjk-xjMWMWk-MWN

Concerns:

- I’ve assumed that the “area” for the molar flow is the same “area” as the catalyst bed. This is likely incorrect.

Correction: Aphysical=Acat+Afluid

Ap=Ac+Af=Ac1+AfAc

AfAc=ApAc-1

Question: what is the ratio of physical (i.e. pipe) area to catalyst area?

Vp=ApLp∝D2L

Vc∝NcR3

Where R is the typical radius of a catalyst particle and Nc is the number of catalyst particle

VcVp=fpacking=4Ncπ3Rc3π4D2L=16NcDc33*8*Dp2L=23NcDc3Dp2L

For Dc catalyst particle diameter, Dp pipe diameter, L catalyst bed length

Roughly:

Average catalyst x-section area:

Vc=Vp23NcDc3Dp2L

Ac≈VcL=Vp23NcDc3Dp2L2=NcπDc36L

ApAc=π4Dp2NcπDc36L=32LDp2NcDc3

AfAc=ApAc-1=1fpacking-1

For fpacking~0.5 this is order 1

Derive for fixed volume from scratch

Fj=Nj=NjVQ=njdVgasdt=ddtnjVg-Vgdnjdt   mols

dm=ρcdVcat

VgasVcat=Vp-VcVc=1fpack -1

dFjdm=1ρcddVcatnjdVgasdt=1fp-11ρcddVcdnjVcdt-Vcdnjdt

dFjdm=1fp-11ρcdnjdt-dnjdt