# Reactor Transient Model

Graham currently has a steady-state model of a plug flow reactor made in Excel. This works well and is capable of solving the configurations that can be expressed as an IVP (i.e. co-flow or isothermal cooling fluid heat exchanger). It can be used for point-wise parameter variation (i.e. select a few cases of interest and run them).

If we want to start doing any of the following, we will need a model implemented in code using either a spatial discretization or an existing IVP solver (i.e. scipy integration library):

- Linearized sensitivity to variations in parameters

- Large (i.e. Monte Carlo style) explorations of parameter variations

Additionally, if we want to start doing any of the following, we will need a spatially discretized model and, if not a true transient model, at least a proper model of the system inertia:

- Any diffusion/mixing

- Steady-state solution stability analysis

- Linearized transfer function (needed for modelling control systems)

# Transient Model of PFR

Governing equations:

∂tρ+∂xρu=0

∂tρu+∂xρuu=-∂xP

∂tρe+∂xρuh=qin

∂tρY+∂xρY=Mρcatj∈rxnνijrj

Start by simplifying the system by assuming ideal gas and no spatial gradients in pressure. Pressure variation will be an input to the system (i.e. one of the parameters we can control). This immediately eliminates one of the equations, because if P is known then with e and Y we can immediately determine density. Therefore, we can eliminate either the continuity or the momentum equation immediately. We need to be careful which we choose, because it might ultimately result in a degenerate mass matrix. To look at this in more detail, write out the equations in mass matrix form:

10uρ0000e0Y0ρ00ρ∂tρueY=M∂tρueY=-∂xρuρuuρueρuY+S

Define F, B, D such that:

F∂tρuTY=∂tρueY,    B∂tρuTX=∂tρuTY,    D∂tPuTX=∂tρuTX

Then we have:

MFBD∂tPuTX=∂tPuTX=-∂xρuρuuρueρuY+S

We know that either the first or the second equation is unnecessary. This means we can eliminate either the first or the second row of M and the corresponding row on the RHS. We want to replace this with a forcing function on pressure:

∂tP=f(t)

This can be added by taking MFBD∈R3x4 and adding a row with a single non-zero element. If we eliminate continuity, the system is:

1000M1FBD∂tPuTX=-∂xf(t)ρuuρueρuY+S

If we eliminate momentum the system is:

1000M2FBD∂tPuTX=-∂xf(t)ρuρueρuY+S

It seems almost certain that eliminating momentum will cause degeneracy since then M2 has a column of all zeros and none of F, B, D couple any of the other variables to velocity. Therefore we go with the option of eliminating continuity. Then:

M1=uρe0Y000ρ00ρ

F converts from internal energy to temperature:

de=cvdT+∑hif-hNfdYi

F=100100000000cvhif-hNf01

B converts from mass fraction to mole fraction:

Yi=XiMiM=XiMi∑XiMi

dYi=1MMidXi-MiM∑Mi-MNdXi

dY=bdX

B=100100000000100b

D converts from density to pressure

dρρ=dPP+dMM-dTT=dPP+∑Mi-MNdXiM-dTT

dρ=ρdPP-dTT+∑Mi-MNdXiM

D=ρP0-ρTρ∑Mi-MNM0I3×3

And then finally:

M=1000M1FBD

Quickly calculating it shows that this is non-degenerate (yay!). So then I can write:

M∂tPuTX=-∂x0ρuuρueρuY+f(t)000+S

In steady state, a different approach makes sense. In that case, continuity dictates that ρu=const. Since ρ is fully determined (by P, T and Y), u is trivial to calculate. In fact, we need to calculate it this way, since with no driving force (pressure gradient), the momentum equation will give a contradictory result.

I’m actually a little worried this problem might appear in the transient model – the steady state solution may not be stable unless we assume some small pressure gradient. It might be a situation where we need to assume pressure is constant for the sake of fluid properties, but allow some gradient for the momentum equation to work…

I really don’t want to go to a proper compressible flow model because of pressure-velocity coupling issues…..

Another option (likely the best one): Work with the lagrangian form of the equation. Manually substitute continuity into the other equations and don’t explicitly solve velocity

ρ∂teY=-ρu∂xeY+S

ρ0cvhif-hNf001∂tρTY=-ρu∂xeY+S

ρ0cvhif-hNf00110001000bρP-ρTρ∑Mi-MNM010001∂tPTX=-ρu∂xeY+S

Define the matrices above as F, B, D respectively, then:

M=100ρFBD

M∂tPTX=-ρu∂x0eY+F(t)+S

I still need a way to calculate u -> only option is algebraically from continuity

∂tρ=-∂xρu=-ρ∂xu-∂xρu

Either need to use a DAE solver or explicitly write it out. Explicitly writing it out may be terrible -> it will move all the spatial gradients into the mass matrix

Plan: Try both approaches

- Include pressure as a state variable and hope the model doesn’t attempt to capture waves (it likely will) and that checkerboarding doesn’t become a problem (again, it likely will)

- Use a DAE solver. Unfortunately this will be much slower since forced to use implicit method and likely don’t have a Jacobian. System looks like:

100eρ0Y0ρFBD100∂tPTY=-∂x(ρu)-∂xρuh+Qin-∂xρuY+wf(t)

I’m going to try the second approach first….

For fixed mdot (ρu) and P:

∂thY=u∂xhY+1ρQinw

For fixed mdot, u=ρuρ and du=-1ρ2dρ

I have dρ=DFB-1dhY. Jacobian of RHS:

∂ϕu∂xϕ+u∂xI+∂ϕsρ

The first term can be written as:

∂ϕu∂xϕ=-1ρ2∂ϕρ∂xϕ=-1ρ2DFB-1∂ϕϕ∂xϕ=-1ρ2DFB-1∂xϕ